Vastu/Mumbai/05/2023/31266-2300505

10/05/126-PRDVS

Date: 10.05.2023

Structural Stability Report

Structural Observation Report of Residential Flat No. 715, 7th Floor, Building No 3, Wing - M, "RNA Court yard Co-Op. Hsg. Soc. Ltd.", Sector - 8, Dr. Baba Saheb Ambedkar Road, Poonam Nagar, Village - Penkarpada, Taluka - Thane, District - Thane, Mira Road (East), PIN - 401 107, State - Maharashtra, Country – India.

Name of Owners: Mr. Kalpesh G. Mehta & Mrs. Neelam Kalpesh Mehta.

This is to certify that on visual inspection, it appears that the structure at " RNA Court yard Co-Op. Hsg. Soc. Ltd." is in average condition and the future life can be reasonably takes under normal condition and with proper periodic repairs & maintenance is about 44 years.

General Information:

| A. | Introduction |  |
| --- | --- | --- |
| 1 | Name of Building | "RNA Court yard Co-Op. Hsg. Soc. Ltd." |
| 2 | Property Address | Residential Flat No. 715, 7th Floor, Building No 3, Wing - M, "RNA Court yard Co-Op. Hsg. Soc. Ltd.", Sector - 8, Dr. Baba Saheb Ambedkar Road, Poonam Nagar, Village - Penkarpada, Taluka - Thane, District - Thane, Mira Road (East), PIN - 401 107, State - Maharashtra, Country – India. |
| 3 | Type of Building | Residential |
| 4 | No. of Floors | Ground + 8 Upper Floors |
| 5 | Whether stilt / podium / open parking provided | One Car Parking Space |
| 6 | Type of Construction | R.C.C. Framed Structure |
| 7 | Type of Foundation | R.C.C. Footing |
| 8 | Thickness of the External Walls | 9” thick brick walls both sides plastered |
| 9 | Type of Compound | Brick Masonry Walls |
| 10 | Year of Construction | 2007 (As per Occupancy Certificate) |
| 11 | Present age of building | 16 years |
| 12 | Residual age of the building | 44 years Subject to proper, preventive periodic maintenance & structural repairs. |
| 13 | No. of flats (Per Floor) | 7th Floor is having 4 Flats |
| 14 | Methodology adopted | As per visual site inspection |

| B. | External Observation of the Building |  |
| --- | --- | --- |
| 1 | Plaster | Not found |
| 2 | Chajjas | Not found |
| 3 | Plumbing | Not found |
| 4 | Cracks on the external walls | Found at some places |
| 5 | Filling cracks on the external walls | Found |
| 6 | Cracks on columns & beams | Found |
| 7 | Vegetation | Not found |
| 8 | Leakages of water in the drainage pipes or water pipes | Found |
| 9 | Dampness external in the wall due to leakages | Found |
| 10 | Any other observation about the condition of external side of the building | The external condition of the building is average. |
| C | Internal Observation of the common areas of the building and captioned premises |  |
| 1 | Beams (Cracks & Leakages) | Found |
| 2 | Columns (Cracks & Leakages) | Found at some places |
| 3 | Ceiling (Cracks & Leakages) | Found at some places |
| 4 | Leakages inside the property | Not Found |
| 5 | Painting inside the property | Normal |
| 6 | Maintenance of staircase & cracks | Normal |

| D | Common Observation |  |
| --- | --- | --- |
| 1 | Structural Audit of the Building Under Bye – Laws No. 77 of the Model Bye Laws (Co-Operative Societies Act / Rules) | As per bye Laws No. 77 of Co-Op. Societies bye Laws under the Act the society shall conduct a Structural Audit of the building of the society as follows |
| 2 | Remark | - The flat under valuation is averagely maintained. At some places plaster is pilled out. - No Structural Audit Report is furnished for the perusal |

| E | Conclusion |
| --- | --- |
| The captioned building is having Ground + 8 Upper Floors which are constructed in year 2007 as per Occupancy Certificate. Estimated future life under present circumstances is about 44 years subject to proper, preventive periodic maintenance & structural repairs. The inspection dated 05.05.2023 reveals no structural damage or deterioration to the building. The building as well as the property is maintained in average condition & will stand future life subject to proper, preventive periodic maintenance & Good structural repairs. Our Observations about the structure are given above. The above assessment is based on visual inspection only. |  |

## Manoj B. Chalikwar

Registered Valuer

Chartered Engineer (India)

Reg. No. CAT-I-F-1763

SBI Empanelment No.: SME/TCC/2021-22/86/3

Actual site photographs

Actual site photographs