Vastu/Nashik/01/2024/006660/2304756                                     31/9-458 -RPBS                                                                              Date: 31.01.2024

Structural Stability Report

Commercial Existing Building "Hotel Kubera" Plot No. 2, City Survey No. 6707, Final Plot No. 190, T. P. S. No. 2, Near Trimbak Naka Road, Old Agra Road, Shivaji Udyan, Village - Nashik, Taluka & Dist - Nashik, Pin Code – 422 002, State - Maharashtra, Country - India belongs to Mr. Deelip Murlidhar Rahane.

This is to certify that on visual inspection, it appears that the structure of "Hotel Kubera" is in normal condition and the future life can be reasonably takes under normal condition and with proper periodic repairs & maintenance is about 40 years.

General Information:

| A. | Introduction |  |
| --- | --- | --- |
| 1 | Name of Building | "Hotel Kubera" |
| 2 | Property Address | Commercial Existing Building "Hotel Kubera" Plot No. 2, City Survey No. 6707, Final Plot No. 190, T. P. S. No. 2, Near Trimbak Naka Road, Old Agra Road, Shivaji Udyan, Village - Nashik, Taluka & Dist - Nashik, Pin Code – 422 002, State - Maharashtra, Country – India. |
| 3 | Type of Building | Commercial |
| 4 | No. of Floors | Basement + Ground + 1st + 2nd Floor / Terrace Floor |
| 5 | Whether stilt / podium / open parking provided | Open Car Parking |
| 6 | Type of Construction | R.C.C. Framed Structure |
| 7 | Type of Foundation | R.C.C. Footing |
| 8 | Thickness of the External Walls | 6’ thick brick walls both sides plastered |
| 9 | Type of Compound | Brick Masonry Walls |
| 10 | Year of Construction | 1990 (As per Full Occupancy Certificate) |
| 11 | Present age of building | 33 years |
| 12 | Residual age of the building | 27 years Subject to proper, preventive periodic maintenance & structural repairs. |
| 13 | No. of Flats | - |
| 14 | Methodology adopted | As per visual site inspection |

| B. | External Observation of the Building |  |
| --- | --- | --- |
| 1 | Plaster | Good |
| 2 | Chajjas | Good |
| 3 | Plumbing | Good |
| 4 | Cracks on the external walls | Not Found |
| 5 | Filling cracks on the external walls | Not Found |
| 6 | Cracks on columns & beams | Not Found |
| 7 | Vegetation | Not Found |
| 8 | Leakages of water in the drainage pipes or water pipes | Not Found |
| 9 | Dampness external in the wall due to leakages | Not Found |
| 10 | Any other observation about the condition of external side of the building | The external condition of the structure is in Average condition |
| C | Internal Observation of the common areas of the building and captioned premises |  |
| 1 | Beams (Cracks & Leakages) | Not found |
| 2 | Columns (Cracks & Leakages) | Not found |
| 3 | Ceiling (Cracks & Leakages) | Not found |
| 4 | Leakages inside the property | Not Found |
| 5 | Painting inside the property | External Wall Plaster and Painting Condition is Good. |
| 6 | Maintenance of staircase & cracks | Good |

| D | Common Observation |  |
| --- | --- | --- |
| 1 | Structural Audit of the Building Under Bye – Laws No. 77 of the Model Bye Laws (Co-Operative Societies Act / Rules) | As per bye Laws No. 77 of Co-Op. Societies bye Laws under the Act the society shall conduct a Structural Audit of the building of the society as follows |
| 2 | Remark | - |

| E | Conclusion |
| --- | --- |
| The captioned Bungalow is having Ground + 1s floor which are constructed in year 1990 (As per Full Occupancy Certificate). Estimated future life under present circumstances is about 27 years’ subject to proper, preventive periodic maintenance & structural repairs. The inspection dated 29.01.2024 reveals no structural damage or deterioration to the building. The building as well as the property is maintained in normal condition & will stand future life subject to proper, preventive periodic maintenance & Average structural repairs. Our Observations about the structure are given above. The above assessment is based on visual inspection only. |  |

Sharadkumar B. Chalikwar

Govt. Reg. Valuer

Chartered Engineer (India)

Reg. No. (N) CCIT/1-14/52/2008-09

Encl: Valuation report.

Actual Site Photographs

Actual Site Photographs