Vastu/Mumbai/01/2025/ 013857 / 2310188
23/13-307 -PRBS
		Date: 23.01.2025

Structural Stability Report

Structural Observation Report of Residential Flat No. 55, 2nd Floor, Wing - D, " Rajdeep Co-Op. Hsg. Soc. Ltd.", Gokhale Road, Village - Naupada, Thane (West), Taluka & District - Thane, PIN - 400 602, State - Maharashtra, Country - India.

Name of Owner: Mr. Vikram Rajendra Jadhav & Mrs. Vedika Vikram Jadhav

This is to certify that on visual inspection, it appears that the structure at "Rajdeep Co-Op. Hsg. Soc. Ltd." is in good condition and the future life can be reasonably taken under good condition and with proper periodic repairs & maintenance is about 15 years.

General Information:

| A. | Introduction |  |
| --- | --- | --- |
| 1 | Name of Building | “Rajdeep Co-Op. Hsg. Soc. Ltd.” |
| 2 | Property Address | Residential Flat No. 55, 2nd Floor, Wing - D, "Rajdeep Co-Op. Hsg. Soc. Ltd.", Gokhale Road, Village - Naupada, Thane (West), Taluka & District - Thane, PIN - 400 602, State - Maharashtra, Country – India. |
| 3 | Type of Building | Residential |
| 4 | No. of Floors | Ground + 4 Upper Floors |
| 5 | Whether stilt / podium / open parking provided | Open Car Parking Space |
| 6 | Type of Construction | R.C.C. Framed Structure |
| 7 | Type of Foundation | R.C.C. Footing |
| 8 | Thickness of the External Walls | 9” thick brick walls both sides plastered |
| 9 | Type of Compound | Brick Masonry Walls |
| 10 | Year of Construction | 1980 (As per Occupancy Certificate) |
| 11 | Present age of building | 45 years |
| 12 | Expected balance lift of the building | 15 years Subject to proper, preventive periodic maintenance & structural repairs. |
| 13 | No. of flats (Per Floor) | 2nd Floor is having 4 Flats |
| 14 | Methodology adopted | As per visual site inspection |

| B. | External Observation of the Building |  |
| --- | --- | --- |
| 1 | Plaster | Normal Condition |
| 2 | Chajjas | Normal Condition |
| 3 | Plumbing | Normal Condition |
| 4 | Cracks on the external walls | Found at some places |
| 5 | Filling cracks on the external walls | Found at many places |
| 6 | Cracks on columns & beams | Not Found |
| 7 | Vegetation | Not Found |
| 8 | Leakages of water in the drainage pipes or water pipes | Not Found |
| 9 | Dampness external in the wall due to leakages | Not Found |
| 10 | Any other observation about the condition of external side of the building | Structural Stability Report from licensed structural engineers not provided for our verification. |
| C | Internal Observation of the common areas of the building and captioned premises |  |
| 1 | Beams (Cracks & Leakages) | Not Found |
| 2 | Columns (Cracks & Leakages) | Not Found |
| 3 | Ceiling (Cracks & Leakages) | Not Found |
| 4 | Leakages inside the property | Not Found |
| 5 | Painting inside the property | Normal Condition |
| 6 | Maintenance of staircase & cracks | Good |

| D | Common Observation |  |
| --- | --- | --- |
| 1 | Structural Audit of the Building Under Bye – Laws No. 77 of the Model Bye Laws (Co-Operative Societies Act / Rules) | As per bye Laws No. 77 of Co-Op. Societies bye Laws under the Act the society shall conduct a Structural Audit of the building of the society as follows |
| 2 | Remark | Structural Stability Report from licensed structural engineers not provided for our verification. |

| E | Conclusion |
| --- | --- |
| The captioned building is having Ground + 4 Upper Floors which are constructed in year 1980 (As per Occupancy Certificate). Estimated future life under present circumstances is about 15 years subject to proper, preventive periodic maintenance & structural repairs. The inspection dated 10.01.2025 of building. The building is well as the property is maintained in good condition & will stand future life subject to proper, preventive periodic maintenance & Good structural repairs. Our Observations about the structure are given above. The above assessment is based on visual inspection only. Separate structural audit from licensed structural engineers is advised to assess exact balance life of structure. |  |

## Manoj Chalikwar

Structural Engineer Licence No. STATE/R/2025/APL/15680

Govt. Reg. Valuer

Chartered Engineer (India)

Reg. No. IBBI/RV/07/2018/10366

Actual site photographs